NEW TOYOTA YARIS – SPECIFICATIONS

| POWERTRAIN | HYBRID 115 | HYBRID 130 |
| --- | --- | --- |
| TYPE | 3-CYLINDER, IN-LINE |  |
| VALVE MECHANISM | DOHC FOUR-VALVE ROLLER ROCKER (IN: MD-VVT, EX: VVT-I) |  |
| FUEL SYSTEM | DIRECT MULTIPOINT INJECTION |  |
| DISPLACEMENT (CM3) | 1,490 |  |
| COMBINED POWER OUTPUT (PS / KW) | 116 / 85 | 130 / 96 |
| POWERTRAIN MG2 MAX TORQUE (NM) | 141 | 185 |
| ENGINE MAX POWER (kW @ RPM) | 68 @ 5,500 |  |
| ENGINE MAX TORQUE (Nm @ RPM) | 120 @ 3,600-4,800 |  |

| HYBRID SYSTEM | HYBRID 115 | HYBRID 130 |
| --- | --- | --- |
| HYBRID BATTERY | LITHIUM ION |  |
| NUMBER OF CELLS | 48 |  |
| RATED VOLTAGE (V) | 177.6 |  |
| BATTERY CAPACITY (3HR) AMP.HR. | 4.3 |  |
| FRONT MOTOR TYPE | PERMANENT MAGNET 3-WAY SYNCHRONOUS MOTOR |  |
| RATED VOLTAGE (V) | 580 |  |
| MAX OUTPUT (DIN HP / KW) | 80 / 59 | 83 / 62 |

| PERFORMANCE | HYBRID 115 | HYBRID 130 |
| --- | --- | --- |
| MAX. SPEED (KM/H) | 175 |  |
| ACC. 0-100 KM/H (SEC) | 9.7 | 9.2 |

| FUEL CONSUMPTION | HYBRID 115 | HYBRID 130 |
| --- | --- | --- |
| WLTP COMBINED CYCLE (L/100KM) | 3.8-4.9 | 3.9-4.3 |
| FUEL TANK CAPACITY (L) | 36 |  |

| CO2 EMISSIONS (G/KM) | HYBRID 115 | HYBRID 130 |
| --- | --- | --- |
| EURO CLASS | EURO 6 AP |  |
| WLTP COMBINED CYCLE | 87-112 | 87-98 |

| CHASSIS |  |
| --- | --- |
| FRONT SUSPENSION | MACPHERSON STRUT |
| REAR SUSPENSION | TORSION BEAM |
| STEERING LOCK TO LOCK MIN. TURNING RADIUS TYRE (M) MIN. TURNING RADIUS BODY (M) | RACK&PINION 2.73 5.2 5.5 |
| BRAKES FRONT REAR | VENTILATED DISC BRAKE WITH FLOATING CALIPER 1-CYLINDER SOLID DISC BRAKE WITH FLOATING CALIPER 1-CYLINDER |
| TYRES | 185/65R15, 195/55R16, 205/45R17 AND 215/40R18 |

| EXTERIOR DIMENSIONS (MM) |  |
| --- | --- |
| OVERALL LENGTH | 3,940 |
| OVERALL WIDTH | 1,745 |
| OVERALL HEIGHT | 1,500 |
| TREAD FRONT | 1,521 – 1,531 |
| TREAD REAR | 1,518 – 1,528 |
| WHEELBASE | 2,560 |
| FRONT OVERHANG | 790 |
| REAR OVERHANG | 590 |

| INTERIOR DIMENSIONS (MM) |  |
| --- | --- |
| LENGTH | 1,845 |
| WIDTH | 1,430 |
| HEIGHT | 1,190 |

| CARGO (DM3 VDA) |  |
| --- | --- |
| CAPACITY (5 SEAT MODE, TO PARCEL SHELF) | 286 |

| WEIGHT (KG) | HYBRID 115 | HYBRID 130 |
| --- | --- | --- |
| KERB WEIGHT MIN | 1,085 | 1,140 |
| MAX | 1,185 | 1,190 |
| TOTAL GROSS MASS | 1,615 |  |